SECTION 15700

M-SERIES™ ALL-IN-ONE HVAC SYSTEM

** NOTE TO SPECIFIER ** MagicPak; heating and cooling solutions.
This section is based on the products of MagicPak, which is located at:
215 Metropolitan Dr.  
West Columbia, SC 29170 
Tel: 866-282-7257
Email: MagicPak.Applications@alliedair.com
Web: www.MagicPak.com  
 [ Click Here ] for additional information.
MagicPak All-In-One Units combine heating and cooling in one compact, easy-to-install unit. Because these are ducted units, they are perfect for apartments, condominiums, assisted living, dormitories, and other multiple occupancy buildings.
Professionals who take pride in a building's appearance and expect reliable performance know that MagicPak units are their best choice.
Available in gas heating/electric cooling, heat pump and electric heating/cooling, MagicPak units' features make them the easiest combination heating and cooling you can specify.

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER ** Delete items below not required for project.

    - Gas heating/ electric cooling packaged units. (MGE)

    - Electric heating/ electric cooling packaged units. (MCE)

    - Electric heat pump packaged units. (MHP)

  - RELATED SECTIONS

** NOTE TO SPECIFIER ** Delete any sections below not relevant to this project; add others as required.

    - Section 15050 - Common Work Results for HVAC.

    - Section 16050 - Common Work Results for Electrical.

  - REFERENCES

** NOTE TO SPECIFIER ** Delete references from the list below that are not actually required by the text of the edited section.

    - ANSI/AHRI 210/240: Unitary Small Air Conditioners and Heat Pumps.

  - SUBMITTALS

    - Submit under provisions of Section 01300 - Administrative Requirements.

    - Product Data: Manufacturer's data sheets on each product to be used, including:

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Installation methods.

    - Shop Drawings and Wiring Schematics: Provide detailed physical layout and wiring schematics for installation.

  - QUALITY ASSURANCE

    - Manufacturer Qualifications: Minimum 5 year experience manufacturing similar products.

    - Installer Qualifications: Minimum 2 year experience installing similar products.

** NOTE TO SPECIFIER ** Include a mock-up if the project size and/or quality warrant taking such a precaution. The following is one example of how a mock-up on a large project might be specified. When deciding on the extent of the mock-up, consider all the major different types of work on the project.

    - Mock-Up: Provide a mock-up for evaluation of surface preparation techniques and application workmanship.

      - Finish areas designated by Architect.

      - Do not proceed with remaining work until workmanship is approved by Architect.

      - Refinish mock-up area as required to produce acceptable work.

  - PRE-INSTALLATION MEETINGS

    - Convene minimum two weeks prior to starting work of this section.

  - DELIVERY, STORAGE, AND HANDLING

    - Deliver and store products in manufacturer's unopened packaging bearing the brand name and manufacturer's identification until ready for installation.

    - Handle materials as recommended by manufacturer to avoid damage.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's recommended limits.

  - SEQUENCING

    - Ensure that products of this section are supplied to affected trades in time to prevent interruption of construction progress.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer: MagicPak, which is located at: 215 Metropolitan Dr.  ; West Columbia, SC 29170 ; Tel: 866-282-7257; Email: MagicPak.Applications@alliedair.com; Web: www.MagicPak.com

** NOTE TO SPECIFIER ** Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions: Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of Section 01600 - Product Requirements.

  - GAS HEATING / ELECTRIC COOLING ALL-IN-ONE UNITS

    - MGE8 M-Series Model Units: Gas Heating / Electric Cooling Single-Package Air Conditioners (SPAC) and Residential Packaged Units Single Phase as manufactured by Allied Air Enterprises LLC, A Lennox International Inc. Company.

      - Capacity:

** NOTE TO SPECIFIER ** Delete capacity for heating and cooling not required.

        - Cooling Capacity: ¾ ton.

        - Cooling Capacity: 1 ton.

        - Cooling Capacity: 1 ½ ton.

        - Cooling Capacity: 2 ton.

        - Cooling Capacity: 2 ½ ton.

        - Cooling Capacity: 3 ton.

        - Heating Capacity: 15 kBTUH.

        - Heating Capacity: 24 kBTUH.

        - Heating Capacity: 36 kBTUH.

        - Heating Capacity: 48 kBTUH.

        - Heating Capacity: 60 kBTUH.

      - Construction:

        - Certified ANSI/AHRI 210/240: Unitary Small Air Conditioners and Heat Pumps.

        - From 12 SEER/11.7 SEER2 to 14 SEER/13.4 SEER2 and 11 EER/10.6 EER2 cooling efficiency.

        - Completely self-contained heating and cooling package.

          - No outside condensing unit.

          - No external refrigerant lines.

          - No separate cooling coil.

          - No separate venting system.

          - No additional drain pan required

        - Pre-wired and pre-charged.

        - Individually metered and controlled.

        - Easy service access.

        - Louver color options to match building facade.

        - Internal vent system (direct vent).

      - Refrigerant System:

        - R410-A Refrigerant.

        - Rifled copper tubing with lanced fin coils.

      - Heat Exchanger:

** NOTE TO SPECIFIER ** Options. Delete material not required.

        - Standard stainless-steel heat exchanger.

      - Cabinet:

        - Galvanized steel cabinet material.

        - Fully insulated cabinet for quiet operation.

** NOTE TO SPECIFIER ** Options. Delete options not required.

        - Wall sleeve as scheduled or indicated.

        - Decorative extruded aluminum louver as scheduled or indicated.

      - Blower:

        - Five-speed permanently lubricated motor for low maintenance, maximum efficiency and wide airflow range.

        - ECM permanently lubricated motor for low maintenance and maximum efficiency.

        - Dynamically balanced blower with resilient motor mounts for smooth and quiet operation.

        - Internal washable filter.

        - Upgrade MERV 8 filter.

      - Controls:

        - Color coded wiring for easy service.

        - Fuse protection on 24V circuit.

      - Installation:

        - Easy to install and service.

        - External utility connections for gas and high and low voltage lines.

        - Louver mounts flush to outside wall.

      - Warranty:

        - 5 year limited warranty on all parts.

** NOTE TO SPECIFIER ** Delete heat exchanger warranty not required.

        - 20 year warranty on stainless steel heat exchanger.

  - ELECTRIC HEATING/ ELECTRIC COOLING ALL-IN-ONE UNITS

    - MCE M-Series Model Units: Electric Heating / Electric Cooling Single-Package Air Conditioners (SPAC) and Residential Packaged Units Single Phase as manufactured by Allied Air Enterprises LLC, A Lennox International Inc. Company.

      - Capacity:

** NOTE TO SPECIFIER ** Delete capacity for heating and cooling not required.

        - Cooling Capacity: ¾ ton.

        - Cooling Capacity: 1 ton.

        - Cooling Capacity: 1 ½ ton.

        - Cooling Capacity: 2 ton.

        - Cooling Capacity: 2 ½ ton.

        - Cooling Capacity: 3 ton.

        - Heating Capacity: 3 kW.

        - Heating Capacity: 5 kW.

        - Heating Capacity: 7 kW.

        - Heating Capacity: 10 kW.

      - Construction:

        - Certified ANSI/AHRI 210/240: Unitary Small Air Conditioners and Heat Pumps.

        - From 12 SEER/11.7 SEER2 to 14 SEER/13.4 SEER2 and 11 EER/10.6 EER2 cooling efficiency.

        - Completely self-contained heating and cooling package.

          - No outside condensing unit.

          - No external refrigerant lines.

          - No separate cooling coil.

          - No additional drain pan required

        - Pre-wired and pre-charged.

        - Individually metered and controlled.

        - Easy service access.

        - Louver color options to match building facade.

** NOTE TO SPECIFIER ** Options. Delete options not required.

        - Optional factory installed ventilation air - up to 15% fresh air.

        - Provide No Electric Heat design.

      - Refrigerant System:

        - R410-A Refrigerant.

        - Rifled copper tubing with lanced fin coils.

      - Cabinet:

        - Fully insulated cabinet for quiet operation.

** NOTE TO SPECIFIER ** Options. Delete options not required.

        - Wall sleeve as scheduled or indicated.

        - Decorative extruded aluminum louver as scheduled or indicated.

      - Blower:

        - Five-speed permanently lubricated motor for low maintenance, maximum efficiency and wide airflow range.

        - ECM permanently lubricated motor for low maintenance and maximum efficiency.

        - Dynamically balanced blower with resilient motor mounts for smooth and quiet operation.

        - Internal washable filter.

        - Upgrade MERV 8 filter.

      - Controls:

        - Color coded wiring for easy service.

        - Fuse protection on 24V circuit.

        - No heat models include 24V output to control external heat source.

      - Installation:

        - Easy to install and service.

        - Louver mounts flush to outside wall.

        - External utility connections high and low voltage lines.

      - Warranty:

        - 5 year limited warranty on all parts.

  - ELECTRIC HEAT PUMP ALL-IN-ONE UNITS

    - MHP M-Series Model Units: Electric Heat Pump Single-Package Heat Pumps (SPHP) and Residential Packaged Units Single Phase as manufactured by Allied Air Enterprises LLC, A Lennox International Inc. Company.

      - Capacity:

** NOTE TO SPECIFIER ** Delete capacity for heating and cooling not required.

        - Cooling Capacity: ¾ ton.

        - Cooling Capacity: 1 ton.

        - Cooling Capacity: 1-1/2 ton.

        - Cooling Capacity: 2 ton.

        - Cooling Capacity: 2-1/2 ton.

        - Cooling Capacity: 3 ton.

        - Heating Capacity: 3 kW.

        - Heating Capacity: 5 kW.

        - Heating Capacity: 7 kW.

        - Heating Capacity: 10 kW.

      - Construction:

        - Certified ANSI/AHRI 210/240: Unitary Small Air Conditioners and Heat Pumps.

        - From 12 SEER/11.7 SEER2 to 14 SEER/13.4 SEER2 and 11 EER/10.6 EER2 cooling efficiency.

        - Up to 8 HSPF heating efficiency.

        - Completely self-contained heating and cooling package.

          - No outside condensing unit.

          - No external refrigerant lines.

          - No separate cooling coil.

          - No additional drain pan required

        - Pre-wired and pre-charged.

        - Individually metered and controlled.

        - Easy service access.

        - Louver color options to match building facade.

** NOTE TO SPECIFIER ** Option. Delete if not required.

        - Optional factory installed ventilation air - up to 15% fresh air.

      - Refrigerant System:

        - R410-A Refrigerant.

        - Rifled copper tubing with lanced fin coils.

      - Cabinet:

        - Fully insulated cabinet for quiet operation.

** NOTE TO SPECIFIER ** Options. Delete options not required.

        - Wall sleeve as scheduled or indicated.

        - Decorative extruded aluminum louver as scheduled or indicated.

      - Blower:

        - Five-speed permanently lubricated motor for low maintenance, maximum efficiency and wide airflow range.

        - ECM permanently lubricated motor for low maintenance and maximum efficiency.

        - Dynamically balanced blower with resilient motor mounts for smooth and quiet operation.

        - Internal washable filter.

        - Upgrade MERV 8 filter.

      - Controls:

        - Color coded wiring for easy service.

        - Fuse protection on 24V circuit.

      - Installation:

        - Easy to install and service.

        - Louver mounts flush to outside wall.

        - External utility connections high and low voltage lines.

      - Warranty:

        - 5 year limited warranty on all parts.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until wall openings and rough-in have been properly prepared.

    - If preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Prepare conditions using the methods recommended by the manufacturer for achieving the best result for the operation under the project conditions.

  - INSTALLATION

    - Install in accordance with manufacturer's instructions. Test for proper operation and adjust as required until satisfactory results are obtained.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

END OF SECTION